CONNECTED DEVICE SECURITY ASSESSMENT

Module 1 — BLE & GATT Foundations

Lab 1 of 6

Where this fits. This is the first skill building lab. You will build a working BLE peripheral and learn to read its GATT table, the very structure you will examine later in the InfiniTime firmware. The characteristics, properties, and permission flags is the foundation for the capstone findings about unauthenticated access to sensitive characteristics.

# Objectives

- Describe the BLE protocol stack and the roles of GAP, GATT, ATT, and SMP.

- Configure advertising and establish a connection between a peripheral and a central.

- Define a GATT service with characteristics and read/write them from a central (the nRF Connect mobile app).

- Read a device's GATT table and state, for each characteristic, what access it permits and what link security it requires.

# What you'll use

- Nordic Developer Academy’s Bluetooth LE Fundamentals, Lessons 1–4 (free, self-paced).

- Exercise code: NordicDeveloperAcademy/bt-fund.

- nRF Connect SDK in VS Code (build/flash) and nRF Connect for Mobile (your central/client).

- Hardware: a Nordic DK (nRF52840 / nRF5340 / nRF52833 / nRF52). Assumes basic C and the nRF Connect SDK Fundamentals install/build steps.

# The Lab

## Part A — Build it

- Lesson 1: Review the BLE stack. Flash the LED Button Service sample, then from the mobile app read the Button characteristic and toggle the LED. Note which layer (GAP/GATT/ATT/SMP) each action exercises.

- Lesson 2: Configure advertising and find your device in a scanner. Record what data your device broadcasts before any connection exists.

- Lesson 3: Achieve communication with the central. Establish a connection and observe the connection parameters.

- Lesson 4: Define or extend a GATT service with at least one readable and one writable characteristic, and exercise both from the mobile app.

## Part B — Analyze it

- Document the complete GATT table of the peripheral you built. for every service and characteristic, record name, UUID, properties (read / write / notify / …), and the permission/security flags set.

- For each characteristic, write one line: what a connected central can do with it, and what link security (none / encryption / authentication) is required first.

- Reflection: if this peripheral streamed health data, which of your characteristics would expose that data to any unpaired peer as currently configured, and which single change would prevent it?

# Deliverable

Fill out the provided report containing: your GATT table, the per characteristic access/security annotations, and your reflection answer. Include one (screenshot or video) from the nRF Connect mobile app showing the discovered services on your device.

Note: No CVSS yet but begin the habit. For each characteristic, label the access it should require given its data sensitivity. In Module 3 you will convert these qualitative judgments into your first full CVSS 3.1 vectors.

# Grading (100 points)

| Criterion | Pts | Form |
| --- | --- | --- |
| Working peripheral builds, advertises, connects, characteristic read/write | 40 | Demo |
| Complete, correct GATT table (all characteristics, properties, flags) | 30 | Report |
| Per-characteristic access & link-security annotations | 20 | Report |
| Reflection links data exposure to a required control | 10 | Report |

# Instructor Notes

- Capstone link: This lab builds the foundational knowledge for Bluetooth protocol stacks that will be used when enumerating for vulnerabilities in InfiniTime firmware.

- Hardware: The build/flash steps need a Nordic DK. This board will be used in modules 1, 3 & part II. The client side needs only a phone.

- Pacing: Nordic estimates ~8–10 h for the full course, we are only doing lessons 1-4.

- Bonus: If you have extra time, look into the other lessons provided by Nordic.