CONNECTED DEVICE SECURITY ASSESSMENT

Module 3 — BLE Security & Pairing

Lab 3 of 6

Where this fits. You move from “is this characteristic protected?” (Modules 1–2) to “is the connection protected, and is the protection actually enforced?” You implement pairing the right way, watch it happen on the air with a sniffer, and write your first full CVSS vector.

# Objectives

- Explain pairing vs bonding, legacy vs LE Secure Connections, and the role of I/O capabilities.

- Distinguish encryption from authentication and “MITM requested” from “MITM enforced.”

- Add pairing to a peripheral and require an encrypted, authenticated link on a characteristic.

- Capture a connection with the nRF Sniffer and show the difference between an unsecured read and a secured one.

- Produce a defensible CVSS 3.1 vector for an unprotected data finding.

# What you'll use

- Nordic Developer Academy BLE Fundamentals, Lesson 5 (Security Fundamentals)

the NordicDeveloperAcademy/bt-fund exercises.

- Your peripheral/tools from Module 1

nRF Connect SDK + a Nordic DK

nRF Sniffer + Wireshark

nRF Connect for Mobile

- The FIRST.org CVSS 3.1 calculator.

# The Lab

## Part A — Secure it (NDA Lesson 5)

- Add pairing and bonding to your Module 1 peripheral. Set the Security Manager parameters bonding, I/O capability, MITM, and LE Secure Connections. Then note what each one requests.

- Mark one characteristic as requiring encrypted and authenticated. Leave another characteristic unprotected for contrast.

- Pair from the phone and confirm the protected characteristic refuses access until the link is paired and encrypted, while the unprotected one does not. (pcap/Screenshot/Video)

## Part B — Watch it & reason (the deliverable)

- Run the nRF Sniffer and capture a connection. Read the unprotected characteristic before pairing (its value crosses the air in the clear) and read the protected one after pairing (encrypted). Save both captures.

- The downgrade probe: have the central present “No Input, No Output” capability and watch which pairing method is negotiated. Does requesting MITM protection actually prevent a Just Works pairing? Record exactly what happens.

- Your first CVSS vector: for the baseline finding “sensitive characteristic readable over an unencrypted link” build a full CVSS 3.1 vector and justify each metric. Fill in:

CVSS:3.1/AV:?/AC:?/PR:?/UI:?/S:?/C:?/I:?/A:? ↑ attack vector — is BLE Network, or Adjacent?

Justify every metric. Pay special attention to AV: a phone app over the internet is AV:N, but a BLE peer must be in radio range, which metric value is that?

# Deliverable

A completed submission report plus your capture files: the two sniffer captures (unsecured vs secured read) with a short explanation of what each shows, the downgrade probe observation, and your first full CVSS 3.1 vector with per-metric justification.

Scoring Vector: This is where the scoring metric becomes a real CVSS 3.1 vector. The reasoning you settle here, especially the attack vector for BLE and the confidentiality impact of unprotected data, you will reuse on the findings through the capstone, where it is graded as severity justification.

# Grading (100 points)

| Criterion | Pts | Form |
| --- | --- | --- |
| Working pairing + a characteristic that enforces encryption/authentication | 30 | Demo |
| Sniffer captures clearly showing cleartext vs encrypted reads | 25 | Report/Captures |
| Downgrade probe observation: requested vs enforced MITM | 20 | Report/Captures |
| First CVSS 3.1 vector, defensible metric by metric | 20 | Report |
| Writeup clarity | 5 | Report |

# Instructor Notes

- Capstone link: the downgrade probe is the exact mechanism behind F-3 (Just Works downgrade) in the Capstone.

- Hardware: this is the most hardware dependent module. Two Nordic DKs plus the nRF Sniffer firmware.

- Pacing: Lesson 5 runs ~2–3 h; the sniffer capture and CVSS work add ~2–3 h. A two week block fits comfortably.